Reducing Wildfire Hazards in the Wildland Urban Interface:

Impacts on Timber Yields and

Best Practices for Stand Management

Executive Summary

Tom Hobby, MBA, MA, MSc.

Royal Roads University, Victoria, BC

April 30, 2008

Funded by

Forest Investment Account

Forest Science Program

Reducing Wildfire Hazards in the Wildland Urban Interface:Impacts on Timber Yields and

Best Practices for Stand Management

Executive Summary

# Purpose and Management Implications

The purpose for the research is to better guide fuels management planning and operations within the Wildland Urban Interface (WUI). There currently lacks a complete guide to address many of the issues of complexity surrounding effective fuels hazard and wildfire risk planning. Forest fuel hazards have been addressed by the Filmon report (2004) Forest Practices Board (Forest Practices Board [FPB] 2006) and by the Association of BC Professional Foresters as a problem within British Columbia and recommended changes to implement fuels reduction treatments within the province.

Integration between wildfire hazard assessments and operational planning of fuels treatments in the WUI can be improved. Forest professionals lack the proper training, tools and understanding to analyze the WUI treatment stands in a holistic manner that identifies the wildfire, other ecological and economic components. This project seeks to link these components together to allow fuels treatment project planners and project managers the ability to better understand the economic feasibility of WUI fuels treatments, the social constraints surrounding community acceptance and potential environmental concerns which may be paramount in implementing operational fuels treatments.

Using the methods and models tested through the research, managers of WUI fuels projects will have improved methods for assessing fuels hazard and risk, and gain a better understanding of the economic feasibility and cost constraints of fuels treatment projects.

One of the challenging issues surrounding WUI fuels treatments is the issue of low merchantable stands, which impede project uptake by forest companies. This research will evaluate and determine whether proposed changes to the current stumpage appraisal system is warranted, and evaluate the potential for tenure amendments to be proposed that will foster innovative solutions for fuels treatment projects.

Project Start Date

April 1, 2007 – March 31, 2009

# Introduction

The BC Rocky Mountain Trench (RMT) has an estimated 250,000 hectares departed from historical fire regimes consisting of heavy fuel loads needing thinning and prescribed burning (RMT ER Steering Comm. 2006). Thousands of hectares are in the wildland urban interface (WUI), generally described as a 2 km buffer zone around municipalities, and pose significant wildfire risk (Filmon 2004). Several municipal initiatives within the RMT region have begun fuels treatments on high risk stands.

Causes and effects of the departure from historical fire regimes due to silvicultural practices and fire suppression in BC have been researched (Filmon 2004, Forest Practices Board [FPB] 2006). Initiatives such as the development of Fire Safe Councils in the US, the Fire Smart program and WUI funding in BC, have begun to address WUI fuels problems. Many reports and initiatives identify the need for commercial opportunities to economically handle forest biomass to reduce wildfire risk (USDA 2003a, USDA 2003b). The US has documented the billions of dollars of wildfire costs (above suppression costs) in terms of lost timber values, environmental restoration, socio-economic costs and property values (USDA 2005) and BC has experienced similar losses, as documented from the 2003 fire season (Filmon 2004).

Silvicultural treatments, followed with prescribed burning are needed to minimize wildfire risk within WUI timber stands (Gray 2005). New guidelines are needed to assess these stands to incorporate fuels treatments into forest management plans. Responses by the Ministry of Forest and Range (MOFR) Protection Branch are addressing the recommendations of the Filmon (2004) report, however implementation is challenging. The MOFR and municipalities have begun treating WUI fuels within the region; however, the scale of treatments needs to be seriously increased to be effective. An increased risk of catastrophic wildfires in the RMT is probable if the scale of fuels treatments is not increased (Taylor et al. 1998).

Problems facing fuels reduction efforts in the WUI include: lack of stand data to accurately model wildfire risk under current stand conditions; lack of integration between crown lands, First Nations reserves and private land owners, making strategies difficult to implement on a landscape level; and challenges to implement fuels reduction via prescribed burning due to smoke related issues. Lack of training by forest managers and registered professional foresters (RPFs) of the available tools and methods to assess wildfire risks has been a barrier to implementing fuels treatments and to meet stand restoration/forest health objectives (FPB 2006). There has also been a lack of knowledge about adequate tools and methods for making economic decisions related to prioritizing stand treatments for fuels removal in BC.

For this project, the researchers assessed the fuels hazard on approximately270 hectares near Cranbrook using Fire Management Analyst Plus® software (Carlton 2005) and determined the economic costs for fuels treatments using Landscape Management System (LMS) software (Rural Tech Inst 2006). These assessments were used to determine wildfire risk under various stand treatments and integrated economic analyses for treatments on a stand/polygon basis. Best practices will be developed and are slated for an expert review at a future fuels management WUI guidebook forum that will result in the development of a WUI fuels treatment guidebook for forest managers, First Nations, and city planners.

This project also includes an evaluation of the current stumpage appraisal and Crown tenures available for implementing WUI fuels reduction projects. These reviews will determine what revisions may make WUI treatment economical for licensees and First Nations.  As this component of the project is continuing over the next year, two reports will be peer reviewed and a single combined stumpage and tenures journal article will be developed that will make recommendations for improving the current stumpage appraisal system and for effectively working with current tenures, or new tenures,  to accomplish WUI fuels treatment objectives.

The stumpage and tenures reports were initiated in light of the recommendations made by the Forest Practices Board ( FBP 2005) and Filmon report ( Filmon et.al 2004) which stated that amendments to current tenures and stumpage calculation methods are needed to encourage the proper treatments of high risk WUI stands and create incentives for industry and First Nations’ participation. Low value timber stands in the WUI are not a priority for licensees as fuels treatments are typically not economical and incentives for industry to co-operate in fuels reduction programs has been limited.

The threat of wildfire risk poses millions of dollars of potential losses within the WUI, affecting timber production, ecosystem health, and personal property. The development of guidelines using proper tools for wildfire hazard and risk assessment and implementation, and an evaluation and recommendations for stumpage and tenure policy changes which create the proper incentives for implementing fuels treatments, is critical to BC forest management and has the potential to save the BC government, First Nations and private land owners millions of dollars. This project provides an excellent opportunity to engage local communities and stakeholders in discussions about wildfire hazard and risk and build capacity within these communities that potentially could lead to successful wildfire fuels reduction projects in the WUI.

# Methodology Overview

## Fuels Hazard Assessment Methodology

The researcher’s tested the proposed hazard assessment and economic evaluation methods using a 270 hectare fuels treatment planning site on St. Mary’s Reserve near Cranbrook, BC, The site has many similarities to that of BC Interior rural communities in terms of wildfire hazard and risk, and the site will serve as a pilot study by which to test the research models. The development of the draft WUI guidebook for fuels reduction assessments and implementation will use this pilot case study to explain and present the methods and outcomes of fuels treatment planning.

Fuel hazard reduction objectives in the WUI include: reducing crown fire initiation and propagation conditions; reducing surface fire intensity; and, creating stand-level wildfire resilience (Graham et al. 2004). Stand level data will be collected for aerial fuel strata which are inventoried including density by species, diameter class, height, and crown base height (Fule et al. 2001). Data will be entered into the Fire Management Analyst Plus® (Carlton 2005) An analysis of Crowning Index (Fiedler et al. 2004), which is the minimum windspeed necessary to initiate crowning, using CrownMass will be used in the development of an aerial fuel thinning regimen. Surface fire intensity and wildfire resilience are functions of pre-thinning fuel load plus post-thinning fuel load plus the probable fire environment (fuel moisture,

windspeed, wind direction, etc.), typically expressed as 95th percentile conditions (Cram et al. 2006). Sensitivity analysis will be used to determine the best post-thinning treatment (pile and burn, broadcast burn, left as is, etc.) relative to both a spatial and temporal measure of risk as

well as suppression success and stand survivability. This same stand-level sensitivity analysis can be developed at the landscape-level but using a different suite of models, in this case FARSITE and FlamMap (Finney 2001, Stratton 2004, Stratton 2006). While the current year’s research did not include data collection outside of the stands slated for treatment on St. Mary’s Reserve, the FARSITE and FlamMap dynamic models will be used by the research team to conduct future analysis with additional data in order to demonstrate the applicability of prioritizing fuel treatment locations that maximize their effectiveness at the landscape level.

## Economic Analysis Methodology

An economic analysis was conducted to determine costs of fuels treatments and allowed for a cost effectiveness evaluation. Using stand level data from fuels inventories, LMS software will be used to estimate the associated costs and benefits for implementing fuels treatments (Resource Technology Institute 2006). LMS is able to model silvicultural thinnings, their associated wildfire risk affects, and calculate a net present value for fuels treatments (Rogers 2003).

## Stumpage and Tenure Analysis Methodology

An evaluation of current stumpage appraisal methods and tenures in the context of fuels treatments within the Southern Interior has been made in reference to the current BC MOFR Interior Appraisal Manual (2004) and the MOFR Interior Log Pricing Manual (2006). Experts from the forest industry, government and First Nations were surveyed in a separate project (BC MOFR unpublished 2007), which is currently under confidential review by the Ministry of Forests and has been provided to the proponent for further research. As the researcher’s for this project became aware of this related project early in the year and were given the privilege of reviewing this report, we have conducted additional research that will be discussed as a brief conceptual overview, while protecting the confidence given to the researchers by the Ministry with this report and data.

# Project Scope

The project scope will focus on determining the best practices for fuels treatment planning and will be focused on the Southern Interior of British Columbia. While the ecosystem types across the province can vary significantly, the guidebook will provide tools that will be applicable, yet with appropriate modifications.

The project will be limited to modeling static fuels hazards and will not focus on the many dynamic and temporal modelling methods that are available. These methods are costly exercises, and take specific expertise that is outside of the project team’s experience. These dynamic and temporal methods are referenced only as suggested methods for WUI fuels treatment planners to consider in this guidebook.

The economic analyses will focus on the current economic alternatives for biomass fibre and not evaluate the many bio-fuels and bio-refining options that may be commercialized in the future. The economic analysis for this study will focus on a biomass bio-energy utilization strategy.

The review of tenures will be limited in the extent that will be applicable to WUI fuels treatments, whereas a more in depth analysis for all fuels treatments outside the WUI will not be addressed here.

Finally, a stumpage review will be looking at the current stumpage for fibre in the Rocky Mountain Trench exclusively, which is based upon the limitations of budget and time, yet is intended to give impetus for other regions to pursue this type of analyses.

# Interim Results and Conclusions

## Developing Hazard Reduction Prescriptions in the WUI

In developing a guidebook for WUI fuels hazard reduction planning and operations, we reviewed four currently accepted methods for developing hazard reduction prescriptions and tested each against the output requirements associated with the three hazard reduction objectives. We also reviewed each method’s ability to: a) incorporate actual field inventory of fuel strata data; b) scale up to landscape fire and fuels management analysis; c) link to economic analysis models to be used in treatment cost analysis; and, d) forecast or predict fuel succession dynamics.  The use of actual field inventory-derived fuel strata data provides a critical level of accuracy and precision that is crucial to fire behaviour and fire effects modeling, and to treatment economics.  Considering the values at risk in the WUI it is prudent to base decisions on the best available data.  Scaling up to the landscape level is critical from a strategic fire management perspective.  Large interface areas present the challenge of where to treat first, how much to treat with limited funds and public acceptance, and how intensively to treat.  Starting with an accurate stand-level analysis allows managers to move to the landscape-level analysis with confidence.  Solid stand-level data also allows for an accurate assessment of the economic condition and potential of the stand.  Treatment economics, cost, product recovery, etc., is critical during periods of limited government funding, shortages in labour, and poor forest products markets. And finally, modeling fuel succession dynamics – how will the fuelbed evolve over time – is based on solid fuelbed inventory coupled with site ecology.  Prescription developers, managers and the public all need to recognize that fuel treatments are forever, not simply a one-time treatment.

The three prescription development methods being used in B.C. include: a) FireSmart; b) the Canadian Fire Behaviour Prediction System; and, c) a system based on intensive stand inventory and analysis using a suite of fire behaviour and fire effects models called Fuels Management Analyst Plus (Carlton 2004).

Method C was investigated for prescription development and management involving the use of a suite of models from Fuels Management Analyst Plus (Carlton 2004), coupled with a number of ancillary models and assessment tools including the spatial fire growth and analysis models Farsite (Finney 1998) and FlamMap (Finney 2006), the fuel succession dynamics model Fire and Fuels Extension to the Forest Vegetation Simulator (Reinhardt and Crookston 2003), and an array of economics models.  Fuel strata inventory for 90 plots, three stands of 30 plots each, on the St. Mary’s Indian Reserve were assessed for potential fire behaviour using the model CrownMass® from the FMAPlus 3.0® suite of fire behaviour and fire effects models.  Stand inventory variables included crown base height, and tree density by species, DBH, and height.  These variables are input to CrownMass®, where the following aerial fuel characteristics are derived: Canopy Base Height, Canopy Bulk Density, and Canopy Fuel Load.  These variables are combined with surface fuel characteristics from a number of established surface fuel models, fuel moisture, slope, and influence of tree density on wind movement (wind friction), to derive a wide range of fire behaviour outputs.  The list of outputs includes: surface fire Rate Of Spread (ROS), Flame Length (FL), Torching Index (TI), Crowning Index (CI), spotting distance, and fire type.

Torching Index is defined as the minimum windspeed necessary to initiate crown fire activity.  Torching itself refers to individual tree or small groups of trees with crowns burning, and is often referred to as passive crown fire.  Crowning Index is defined as the minimum windspeed necessary to propagate a crown fire (Fiedler et al. 2004); also referred to as active crown fire.  The index includes

three general hazard levels: low, moderate, and high, that conform to very high winds necessary to propagate a crown fire (low hazard), moderate windspeeds necessary to propagate a crown fire (moderate hazard), and low windspeeds necessary to propagate a crown fire (high hazard).  The actual windspeed ranges are: low = >80 km/h (50 mph); moderate = 40 to 80 km/h (25 to 50 mph); and high = <40 km/h (<25 mph).

St. Mary’s Reserve – Fuels Hazard Assessment

An analysis of the St. Mary’s fuels data focused on potential crown fire behaviour by block and individual plot and the level of treatment necessary to reduce crown fire behaviour.  Further analysis of landscape-level fire and fuels planning and fuel succession dynamics are possible based on the type and accuracy of fuels data collected to date.

Fuel strata data by block and plot were analyzed on 3 proposed logging blocks, which ranged from 57 to 123 hectares each.  Plots were established on approximately 20 hectares for each block and the results are as follows. All plots in Block 1 exhibit passive crown fire at very low windspeeds; an architect of low CBH and the tall grass surface fuel type. Torching Index would be defined as high.  Twenty-seven of the 30 plots in Block 1 exhibit high CI, while two are moderate, and one is low.  The consistently high CI is mostly attributed to high Canopy Bulk Densities throughout this stand type.

Block 2 exhibited more variability in CBH than Block 1, however, all plots still rate out as high TI.  Of the 29 plots in Block 2, 25 rate as high CI, while three rate as moderate, and one rated as low.  Torching Index in Block 3 is similar to both Block 1 and 2, with all plots rated as high.  Dissimilarly, all plots in Block 3 rate as high CI with some of the lowest windspeeds necessary to propagate active crown fire of the three blocks.

Modelling in FMA was conducted with several thinning prescriptions to reduce the CI and TI to a low level at windspeeds >80 km/h (50 mph) to propagate a crown fire. This thinning exercise was used to be the input requirements for the next section, which included an economic analysis of the thinning prescriptions and were based on both a fuels hazard reduction objective and a silvicultural prescription that enhanced ecosystem values that are consistent with the ecosystem restoration objectives within the Rocky Mountain Trench.

Economic Analysis

The three proposed fuels treatment blocks on the St. Mary’s Reserve were analyzed using Landscape Management System software (Rural Technology Institute 2006) and post processing analyses were developed to incorporate the fuels treatment scenarios within the context of differing products for timber, pulp and biomass.

This exercise was intended to use innovative tools by which to model wildfire fuels hazards and economic potential for fuels treatments in one suite of software. The researchers used the forest fuels inventory data inputs to build a tree list within LMS by which an economic analysis for the 3 cutblocks covering 270 hectares was completed. It should be noted, that a regular timber cruise had been conducted in 2002 for the proposed site, but this analysis did not allow for the biomass analysis to be modeled as was done in LMS. As the current LMS models were based on 90 plots covering 60 hectares within the 270 hectare block, further modelling for the 270 hectare area using  nearest neighbour imputation models could be conducted to test the accuracy of the LMS estimates and but would be useful for estimating fuels hazards, timber, pulp and biomass on a landscape level.

The parameters for modeling stand thinning prescriptions were calculated using the following metrics: First, due to the low merchantability of these stands, an inventory assessment of the tree merchantability within each plot greater than 5 cm was made. Trees were determined to be either merchantable or unmerchantable based on visual assessment and using general parameters for

quality measures such as percent sweep  >10 degrees, twist > 25% , broken tops or other notable damage. These results were included in the post processing model that estimated the quantities of biomass, pulp and timber volumes. In addition, it should be noted that the ponderosa pine was deemed suitable for pulp chips, whereas the Douglas fir, was either selected as a saw log (trees > 17.5 cm), or  as biomass (trees less < 17.5 cm).

Further, the bucking algorithm used in the model was as follows:  Trees >17.5 cm were treated as saw logs unless determined unmerchantable; trees from 10 cm – 17.5 cm were treated as pulp logs; trees < 10 cm were treated as biomass in general but with a minimum diameter of 5 cm.  Any trees below this threshold was estimated to be slashed, masticated or burned in follow up prescribed fire.

Harvest Model Results

The three blocks modeled within the total 270 hectare site on the St. Mary’s Reserve were modelled to estimate saw log volume (Table 1), pulp volume (Table 2) and biomass volume (Table 3) using the site logging plan with two silvicultural prescriptions.  In the tables, treatment one represents an open range prescription that leaves a residual of 5 mature trees per hectare, and a maximum stocking of less than 30 stems per hectare (SPH).  Treatment two represents an open forest prescription with a target of 30 mature trees per hectare and a maximum stocking of 250 SPH.

Table 1 – Saw log Volume by Block and Treatment Type

Table 2 – Pulp Volume by Block and Treatment Type

Table 3 – Biomass by Block and Treatment Type

The economic values for these blocks as a single fuels management project were analyzed using a cost benefit analysis framework. Here, the base case was evaluated using several parameters that included: current market prices for ponderosa pine and Douglas fir saw logs, and pulp chips. A proposed scenario 2 analysis was also conducted to determine the value of biomass and for

developing a local ponderosa pine market as these logs are currently not desirable in the local mills and are either exported to the U.S. or utilized as pulp.

Under the base case, current market conditions for saw logs, pulp prices and no market for biomass in the region led to a net negative income of (-$48,611) or a total cost/loss per hectare of $810 for the treatment area.

When evaluating the same 3 cutblocks in a second scenario the following assumptions were made: 1) a local market for ponderosa pine logs was established; 2) there was a market for hog fuel that would utilize the biomass produced from the logging residues for producing electricity; 3) pulp chips were made from only ponderosa and lodgepole pine, as these chips are desirable, whereas Douglas fir chips are not in demand in the chip market. Under the second scenario, the total net income for saw logs, pulp chips and biomass, yielded an estimated net logging income of $170,234 from the 270 hectares modelled, representing a net income of $631 per hectare. When WUI fuels assessment planning, along with logging, and follow up slashing and prescribed burning were added, the total costs for fuels treatment was estimated for the case study area to cost $380 per hectare, yielding a net positive income for the entire fuels reduction treatment of $67,652 or $251 per hectare.

From these results, WUI fuels reduction planners will be able to evaluate new WUI fuels treatment proposals with a new paradigm in mind as they will be aware of the appropriate fuels data to collect and models to effectively use. This fuels hazard assessment model at the stand level is more accurate than previous models used by fuels treatment planners. Using LMS with post output processing allows for an estimation of timber, pulp and biomass produced by removing the fuels via logging and thinning, which has not been done to date by WUI fuels treatment planners. Using LMS also allows for an economic assessment to be made on a stand level that can be customized to estimate current fibre production along with  a projected project net income statement derived from using current log, chip and biomass prices, harvest, chipping, grinding, and hauling costs.

Future work to develop a stand level classification scheme and subsequent polygon modelling methodology to extend the fuels hazard assessment and economic models to the landscape level has yet to be tested. By extending the research to allow for larger landscape level analyses to be conducted, this would hypothetically allow a complete WUI fuels treatment plan to be developed that is an accurate assessment of fuels hazard and allows for developing an economic estimate of average net income, or costs across the landscape, which are needed to attract investment. For smaller scale, stand level analyses of less than 300 hectares, the methods used here for data collection and analysis may be appropriate tools to use that are accurate, cost effective and relatively easy to use. However, the costs of collecting field data as has been done in this case would be costly to extend in a landscape context and alternative approaches should be explored while not risking the loss of accurately assessing the fuels hazard.

# Stumpage and Tenures Review

The Ministry of Forests and Range began a review of the issues surrounding the challenges faced by forest managers, major licensees, and proponents of ecosystem restoration activities in the Rocky Mountain Trench. As there has been a formal ecosystem restoration plan in place for many years (BC Ministry of Forests 2006), the implementation of the ecosystem restoration plan has been challenging in the region due to the current stumpage assessments in particular for ponderosa pine and low grade Douglas fir, which are predominant species in the interior Douglas fir (IDF) ecosystem. While the focus of the Ministry’s report was on ecosystem restoration, the analysis and data collected to write this report was very similar to the interests of the WUI fuels treatment issues, which mainly have been inhibited by low merchantability of WUI timber in the Trench and few tenures by which the WUI fuels treatments can be operationalized.

The Ministry contracted a review of the stumpage by conducting interviews with major and small licensees in the region in order to evaluate whether the associated claims of inappropriate stumpage calculations were justified.  While the early results of this report is still under review by the Ministry and which was allowed by the Ministry for the principal investigator to review, there has been a critique of this report and dialogue with the lead author over the past year. In addition, additional data has been collected that will be used to further evaluate the findings of the Ministry’s report.  Once released by the Minister, the subsequent review and comments along with the researcher’s independent analysis of the issues will be made in separate reports to the Ministry of Forests and Range that supports innovative ways to work within the current policy framework, or new policy proposals to encourage WUI fuels treatment operations.

Literature Cited

Agee, J.K., and C.N. Skinner. 2005. Basic principles of forest fuel reduction treatments. Forest Ecology and Management. 211. 83-96.

Agee, J.K., Bahro, B., Finney, M.A., Omi, P.N., Sapsis, D.B., Skinner, C.N., van Wagtendonk, J.W., Weatherspoon, C.P., 2000. The use of shaded fuelbreaks in landscape fire management. Forest Ecology and Management. 127, 55–66.

Bachmann, A. and B. Allgower. 2000. The need for a consistent wildfire risk terminology. In: L.F. Neuenschwander, and K.C. Ryan (tech. edits.). Proceedings from the Joint Fire Science Conference and Workshop, “Crossing the millennium: integrating spatial technologies and ecological principles for a new age in fire management.”  June 15-17, 1999. Boise, ID. University of Idaho and the International Association of Wildland Fire.

B.C. Ministry of Forests and Range 2007. Harvesting Timber to Restore Fire Maintained Ecosystems: An Assessment of Issues in the Rocky Mountain Trench. Unpublished report.

Ministry of Forests and Range Interior Log Pricing. Jun 2006.

http://www.for.gov.bc.ca/hva/timberp/infopapers/MPSInterior.pdf  (accessed Dec 18, 2006).

Ministry of Forests and Range Interior Appraisal Manual, Nov 2004. http://www.for.gov.bc.ca/hva/manuals/interior/2006DecMaster_17.pdf (accessed Dec 18, 2006).

Carlton, D. 2004. Fuels Management Analyst Plus ver. 3.0. Fire Program Solutions L.L.C. Estacada, OR.

Feidler, C.E., Keegan, C.E. III, Woodal, C.W., and T.A. Morgan. 2004. A strategic assessment of crown fire hazard in Montana: Potential effectiveness and costs of hazard reduction treatments.  U.S. Dep. Agric. For. Serv. Gen. Tech. Rep. PNW-GTR-622. Portland, OR.

Filmon, G., Leitch, D., and J. Sproul. 2004. 2003 firestorm provincial review. Victoria, BC.

Finney, M.A. 2006. An overview of FlamMap fire modeling capabilities. In: Andrews, P.L. and B. Butler. comps. 2006. Fuels management – how to measuring success conference proceedings. U.S. Dep. Agric. For. Serv. Proceedings. RMRS-P-41. Fort Collins, CO.

Finney, M.A. 1998. FARSITE: fire area simulator – model development and evaluation. U.S. Dep. Agric. For. Serv. Res. Pap. RMRS-RP-4 Revised. Fort Collins, CO.

Forestry Canada Fire Danger Group. 1992. Development and structure of the Canadian Forest Fire Behaviour Prediction System.  Forestry Canada Science and Sustainable Development Directorate. Information Report ST-X-3. Ottawa, ON.

Graham, R.T., McCaffrey, S., and T.B. Jain. 2004. Science basis for changing forest structure to modify wildfire behaviour and severity. U.S. Dep. Agric. For. Serv. Gen. Tech. Rep. RMRS-GTR-120. Fort Collins, CO.

Ministry of the Auditor General. 1998. Managing interface fire risks. Victoria, BC.

National Fire Protection Association. 2003. Structure losses by decade. Baltimore, MD.

Partners in Protection. 2003. FireSmart: protecting your community from wildfire. Edmonton, AB.

Rogers, Heather K., Investigation of Alternative Fuel Removal Strategies. Thesis. University of Washington. 2003.

RMT Ecosystem Restoration Steering Comm. Fire Maintained Ecosystem Restoration in the RMT. Blueprint for Action. 2006. URL: http://www.trenchsociety.com/publications.php?ID=1 (accessed April, 2008 )

Rural Technology Institute. Landscape Management System. 2006. URL: http://lms.cfr.washington.edu/ (accessed December 18, 2006).

For more information, contact

The Centre for Non-Timber Resources.

2005 Sooke Rd. 
Victoria, BC V9B 5Y2

250-391-2600 x4328

tom.hobby@royalroads.ca